AIAA SciTech 2021

Abstract submission – Oral presentation

Title: Evaluation of CFD as a Surrogate for Wind-Tunnel Testing – Experimental Uncertainty Quantification (oral only)

Authors: Heather P. Houlden (ViGYAN, NASA Langley Research Center), Erin Hubbard (Jacobs, NASA Glenn Research Center)

A series of wind tunnel tests are being performed at the Unitary Plan Wind Tunnel at Langley Research Center to assess the validity of using computational fluid dynamics (CFD) as a surrogate for wind tunnel testing. In order to make proper comparisons, uncertainties in CFD results and experimental data must be well understood. The material presented will highlight the methods, assumptions, and elemental inputs used to achieve experimental uncertainty estimates for several variables of interest.

The work performed to date has focused on gaining insight into random uncertainty via statistical analysis of repeat data and systematic uncertainty via Monte Carlo propagation analysis. These methods were combined using a second-order Monte Carlo propagation, resulting in a probability box (P-box) for each variable and at all conditions of interest. Figure 1 provides an example of one such P-box, showing the experimental uncertainty in a locally determined dynamic pressure (QC10) for one of many tunnel conditions evaluated during the Flow Survey experiment. This particular condition (Condition 27) has nominal set points of Mach number = 3.85, Reynolds number = 3x106 ft-1, total pressure = 5160 psfa, and total temperature = 150°F. The uncertainty in dynamic pressure at this condition is now well defined by this plot. Armed with this experimental data uncertainty, meaningful comparisons can be made with computational results, when their associated uncertainties are also considered.

Figure 1: Probability box for a local dynamic pressure, QC10, at Condition 27