Parametric Studies of Human Mars Entry, Descent, and Landing Systems

Jamshid Samareh

This paper will present a parametric analysis for entry, descent, and landing (EDL) concepts, enabling rapid systems assessment and tradespace exploration. The entry system uses a hypersonic inflatable aerodynamic decelerator (HIAD) technology. The baseline system includes elements for Mars aerocapture (AC) and EDL segments of the mission.

The mission concept of operations (ConOps) begins at Mars arrival in a polar inclination. After performing an aerocapture (AC) maneuver into a pre-defined Mars parking orbit, the AC HIAD is jettisoned. The rest of the vehicle stays in the parking orbit for up to one year. The EDL sequence starts with a deorbit burn at the apoapsis of the parking orbit. After hypersonic entry with the EDL HIAD, the entry system uses a supersonic retropropulsion maneuver to slow the vehicle for the descent and landing segments of the mission. The vehicle will maintain a constant velocity of 2.5 m/s for 5 seconds prior to landing.

The system includes a Mars Ascent Vehicle (MAV), Mars Descent Module (MDM), and two HIADs. The MDM includes a primary structure, tanks, engines, and radiators. The primary structure is an aluminum-lithium (Al-Li) cruciform design similar to the structural design of the Apollo Lunar Module. The cruciform planform layout results in four outer bays, with adequate volume in the corners between outer bays to package four landing gears. The central bay is reserved for packaging the MAV and the recessed MAV engines. Two of the outer bays accommodate main propellant tanks, with one LOX and one CH4 tank in each bay. The two remaining outer bays each house four rocket engine systems and associated support structure.

Each HIAD comprises an inflatable structure, flexible thermal protection system, gas, and gas generators. The HIAD design used in this study is a stacked-toroid concept with pairing loop straps and radial/chevron straps.

The baseline system lands a 20-t payload on the Mars surface. It is assumed the vehicle arrives at 6.2 km/s relative velocity at 90° inclination and is captured to a 1-Sol parking orbit.

The baseline design includes many assumptions such as margins, arrival state, ConOps options, parking orbit, physical dimensions, propellant options, and technology concepts. The impact of these parameters are quantified through systems-level sensitivity analyses, which capture the global impact—not at a component level—but at the systems level. The systems-level sensitivities expose major design drivers and importance of each assumption for a design.

Through tradespace exploration, a wide range of systems parameters are examined and compared for several feasible design options. Studies have been completed for the following input parameters: payload mass, propellant options, AC/EDL ballistic coefficient, lander thrust to weight ratio (T/W) (surrogate for the maximum EDL g’s), engine specific impulse (Isp), parking orbit, and inclination.

The final paper will present and discuss the parametric approach used in the study. It will also include the results of recent systems analyses, sensitivity analyses, and tradespace exploration.