# Performance of a broad-band, high-resolution, transition-edge sensor spectrometer for x-ray astrophysics

S. J. Smith, J. S. Adams, S. R. Bandler, S. Beaumont, J. A. Chervenak, E. V. Denison, W. B. Doriese, M. Durkin, F. M. Finkbeiner, J. W. Fowler, G. C. Hilton, R. Hummatov, K. D. Irwin, R. L. Kelley, C. A. Kilbourne, M. A. Leutenegger, A. R. Miniussi, F. S. Porter, C. D. Reintsema, J. E. Sadleir, K. Sakai, D. S. Swetz, J. N. Ullom, N. A. Wakeham, E. J. Wassell, M. C. Witthoeft, L. R. Vale.

ABSTRACT: Future x-ray astrophysics experiments require multiplexed readout of high-fill-factor, kilo-pixel arrays of transition-edge sensors (TESs), with very high spectral resolution over a broad range of energies. In this paper we report on a prototype kilo pixel array of Mo/Au TESs read out with 8-column by 32-row time-division multiplexing (TDM). This system is being used to demonstrate the critical detector and readout technology for ESA’s Athena X-IFU, and when complete will used to laboratory astrophysics experiments. Our array and TDM readout has demonstrated a combined energy resolution, including > 200 pixels, of: 1.95 eV for Ti-K (4.5 keV), 1.97 eV for Mn-K (5.9 keV), 2.16 eV for Co-Kα (6.9 keV), 2.33 eV for Cu-Kα (8 keV), 3.26 eV for Br-Kα (11.9 keV). The 1 sigma error is  0.01 eV for all spectra. These results meet the broad-band resolution requirements for X-IFU.