Mass Mitigation in Structural Designs Via Dynamic Properties

The reported efforts are focused on mitigating unnecessary mass in aerospace hardware via leveraging dynamic strength of materials and the frequency dependency of strain. Analytically predicted dynamic responses of structures are often applied as static loads in stress analyses that ultimately dictate the weight of a structural design. Assuming a dynamic response is a static load and then comparing resulting stress predictions to a static strength property is a long-standing engineering practice. Doing so is known to be, or is assumed to be, conservative. However, little indication of the order of magnitude of embedded conservatism has been identified. NASA/MSFC efforts in 2011, 2019, 2020, and now in 2021 have begun to qualitatively show the order of magnitude of that conservatism. A quick turnaround engineering method is pursued to leverage the subject facets of physics for the purpose of decreasing the weight of flight hardware. Tests performed using simple beams and significant observations are described.