# MEDLI2: Ablator Models for Flight

Joshua D. Monk, Jay D. Feldman, Milad Mahzari and Todd R. White

NASA Ames Research Center, Moffett Field, CA 94035, USA

Dinesh K. Prabhu and Hannah Alpert

AMA Inc. NASA Ames Research Center, Moffett Field, CA 94035, USA

During the entry of Mars Science Laboratory (MSL) the heatshield was equipped with the instrumentation suite Mars Entry, Descent, and Landing Instruments (MEDLI).  In-depth thermocouple (TC) data was used to reconstruct the surface heating[1,2].  Discrepancies between MEDLI’s flight heating data and predicted thermal response and recession[3] led to a campaign to characterize and test Mars2020 and MEDLI2 (Mars Entry, Descent, and Landing Instrument 2)[4] flight-lot thermal protection materials (TPS) at relevant ground test conditions.

This paper covers the development and validation of flight-lot material response models for the heatshield material Phenolic Impregnated Carbon Ablator (PICA) and the backshell material SLA-561V (Super Lightweight Ablator).  Virgin and char properties such as thermal conductivity and emissivity were updated as a function of temperature and pressure in the MEDLI2 specific material response models.  Other material properties were characterized and determined to be similar to prior TPS material measurements used for the heritage models.  MEDLI2 also investigated the impact of the protective coating that was applied to MSL and Mars2020’s PICA heatshield to reduce the particulate matter shedding.  Evidence from arc jet experiments on coated PICA showed that although there were no adverse effects on the performance of the TPS, there were differences in the in-depth temperatures and reduced recession compared to material response predictions of uncoated PICA.[5,6].  Fully Implicit Ablation and Thermal response (FIAT)[7] material response simulations were conducted with the heritage and MEDLI2 flight-lot material models.  FIAT-predicted in-depth temperature responses and recession predictions were compared to flight-lot certification ground-test arc jet test data and MEDLI2 thermocouple data.

# References

[1]  White, Todd R., et al. "Post-flight Analysis of Mars Science Laboratory’s Entry Aerothermal Environment and Thermal Protection System Response." 44th AIAA Thermophysics Conference. 2013.

[2]  Mahzari, Milad, et al. "Inverse estimation of the mars science laboratory entry aeroheating and heatshield response." Journal of Spacecraft and Rockets 52.4 (2015): 1203-1216.

[3]   Cruden, Brett A., et al. "Radiative heating during mars science laboratory entry: Simulation, ground test, and flight." Journal of Thermophysics and Heat Transfer 30.3 (2016): 642-650.

[4]  Hwang, Helen, et al. "Mars 2020 entry, descent, and landing instrumentation (medli2)." 46th AIAA Thermophysics Conference. 2016.

[5]  White, Todd, et al. "The Entry Descent and Landing Instrumentation (MEDLI2) Suite for the Mars 2020 Mission." (2019).

[6]	Bessire, Brody K., et al. "Progress Towards Modeling the Mars Science Laboratory PICA-NuSil Heatshield."  The 16th International Planetary  Probe Workshop, Oxford, UK, (July 2019).

[7]  Chen Y.K., (2021), “Multi-dimensional Thermal Response Analysis for PICA/CV-1144-0 in HyMETS Environments. NASA TM publication pending.

[8]	Chen, Y-K., and Frank S. Milos. "Ablation and thermal response program for spacecraft heatshield analysis." Journal of Spacecraft and  Rockets 36.3 (1999): 475-483.

Materials Scientist, Thermal Protection Materials Branch

Project Manager, Thermal Protection Materials Branch

Aerospace Engineer, Entry Systems and Vehicle Engineering Branch, and AIAA Member

Project Manager, Entry Systems and Vehicle Engineering Branch, and AIAA Member

Senior Aerothermodynamicist, Aerothermodynamics Branch, and Associate Fellow AIAA

Systems Engineer, Entry Systems and Vehicle Engineering Branch