# A Model-Based Systems Engineering Journey to Developing a Concept of Operations

## Abstract

Starting in 2017, NASA’s Human Research Program (HRP) Exploration Medical Capability (ExMC) element began a systems engineering transition from traditional, document-centric development to model-centric development when defining its foundation medical systems. These foundation medical systems define a Concept of Operations (ConOps) and identify the generic requirements for a medical system based on assumptions about a generic crew and mission environments and guidance from NASA standards (e.g., Medical “Levels of Care”). By making the transition, ExMC intends to improve communication among stakeholders about foundation medical system requirements and content. In addition, this transition will enable ExMC to lower both development and crew treatment risks for future, mission-specific medical systems.

ExMC followed a Model Based Systems Engineering (MBSE) paradigm when developing the foundation medical systems. A model-based approach provides several advantages over a traditional, document-centric approach. First, when Systems Engineers (SE) develop diagrams in a model using a standard modeling language, they produce information dense pictures that facilitate understanding much more efficiently with less room for misinterpretation than text. Second, due to the evolving nature of projects, documentation becomes out of date the minute it is published. This can result in people making decisions based on information that is no longer current, especially if they are referencing a locally-stored copy of a document. A model, on the other hand, is always up to date with the latest approved changes and information. It serves as a single point of truth. Third, a model-centric approach centralizes all important information in one place. Rather than having to flip through separate ConOps documents, design specifications, requirements specifications, and the like to coordinate information, a model captures the content in one, integrated spot. This integration makes tracing information from end-to-end easier with greater reliability.

The ExMC Systems Engineering Lifecycle follows a well-defined process. ExMC Systems Engineers perform all major steps of the process, regardless of the development methodology. One of the first steps in the process is developing the ConOps that describes the operation of the system from the point of view of the users. It includes a list of the users and their needs, the goals of the medical system, key assumptions about the system, and definitions of the medical system’s operational environments. For this development effort, ExMC chose to replace the traditional text-based ConOps document with a model. While the decision to change the development workflow was not difficult, implementing the structural and organizational workflows were. It required showing ExMC’s users, most of whom are not Systems Engineers, how the information they require would be presented in the model and to gain their acceptance of this approach.

This paper documents key lessons learned during the ConOps transformation by focusing on how the model represents information, the agile workflow used by SEs when developing the model and how it integrates into a project plan, how leadership influenced key users to accept the transformation, and how the users interact with the model information.