Algorithmic Classification of Raman Spectra Biosignatures: Improving Life Detection Confidence

Tao Sheng1,2, Aarya Mishra3,4, Jesse B. Murray5, Sunanda Sharma6, Diana Gentry7

NASA Ames Research Center - Space Life Sciences Training Program (SLSTP) / KBR Inc.1, University of Pittsburgh2, NASA Ames Research Center - Volunteer Internship Program (VIP)3, University of San Francisco4, University of Oxford5, Massachusetts Institute of Technology6, NASA Ames Research Center7

Abstract

“Agnostic” biosignatures – indicators of life (or the absence of life), independent of a particular biochemistry – are increasingly considered a high standard for life detection. The Ladder of Life Detection (2018) called for investigating how combinations of independent and different potential biosignatures affect confidence. To address this gap, statistical classification of elemental abundances, isotopic fractionation, and reflectance spectroscopy (VNIR) has been implemented. Raman spectroscopy, highly desirable due to its wide availability, has the potential to improve this predictive power. This work implemented biosignature classification algorithms on Raman data alone, in preparation for combination with the other data types.

Raman spectroscopy data was collected from published databases and papers as part of a manually curated dataset of “indicative” and “non-indicative of life” samples. These currently include 61 non-indicative samples (meteorites, magnetite); 3 indicative living samples (bacteria); 20 indicative non-living samples (chalk, bone); and 12 indicative mixed (with non-indicative material) samples (soil, microbial mats). Laboratory work is ongoing to characterize additional samples, particularly a greater breadth of mixed systems. Spectra were interpolated, filtered with the Savitzsky-Golay filter, and de-noised. For a preliminary examination, agnostic features were manually extracted including mean intensity, number of peaks, and mean peak width. Different peak prominences and filtering polynomials were used to refine features. Classification algorithms were implemented: k-nearest neighbors (KNN), logistic regression (LR), linear support vector machines (SVM), random forest (RF), Gaussian naïve bayes (GNB).

Lastly, Monte Carlo simulations on 1,000 50%-train-test-splits were used to validate classification performance and feature significance. The preliminary feature set achieved its highest AUC of 0.52 with LR, with no strongly discriminatory features. Work to improve feature extraction, such as through deep learning with back propagation, is planned. In future work, the Raman data will be combined with the other data types, and potentially new data types such as enantiomeric excess.

This project was partially supported through the NASA Ames Project EXcellence (APEX) incubator program.