APH Experiments: Germination and Seedling Establishment Methods

O Monje, JT Richards, LE Spencer, NF Dufour

Plant experiments during ground studies were conducted in the Advanced Plant Habitat (APH) Science Carrier (SC) for ensuring high germination rates and vigorous seedling establishment. The methods constitute a mixture of environmental and planting factors that control moisture and humidity during early seedling establishment. The APH uses granular media that is fertilized with slow release fertilizer pellets. Seeds were sanitized and their germination was tested prior to use in the SC. Seeds were glued in place with guar gum in different wicking materials. It was found that seedlings establishment was reduced if the chamber was too dry or if salt buildup near the seeds was permitted. Thus, germination was carried in media that was flooded and drained, in a chamber held at 80% relative humidity at a low wind speed of 0.3 m/s. After a week the humidity was lowered to 60% RH and wind speed was increased to 0.6 m/s. The germination of different plant species were found to respond differently to germination wicks or to gauze, as well as fertilization rates. Several methods for growing Arabidopsis, wheat, radish and peppers in the APH SC will be presented.