INTRACRANIAL EFFECTS OF ARTIFICIAL GRAVITY: A 3T MRI STUDY

L. A. Kramer1, K. H. Hasan1, X. Zhang1, B. R. Macias2, K. Marshall-Goebel3, S. S. Laurie3, and E. M. Bershad4

1University of Texas, McGovern Medical School, 6431 Fannin Ave, Houston, Texas. Larry.A.Kramer@UTH.TMC.EDU, 2NASA, Houston, TX, 3KBR, Houston TX, and 4Baylor College of Medicine, Houston TX

INTRODUCTION

Spaceflight associated neuro-ocular syndrome (SANS) is characterized by the development of optic disc edema, posterior globe flattening, choroidal/retinal folds and hyperopic refractive errors1. SANS is hypothesized to be a result of headward fluid shifts that invariably occurs in the microgravity environment. As a countermeasure, artificial gravity (AG) through centrifugation has been proposed to reduce this headward fluid shift, however there is no current proof of benefit. The goal of this study was to determine if the application of AG can prevent or reduce known changes in brain volumetry, internal carotid artery (ICA) stroke volume and cerebral spinal fluid (CSF) flow velocity that occurs during simulated chronic headward fluid shift using head down tilt bed rest (HDTBR) methodology2 as an indicator of countermeasure efficacy.

METHODS

Healthy volunteers were recruited for an IRB approved HDTBR study performed at the German Aerospace Center in Cologne, Germany. Strict six-degree HDTBR was used as a spaceflight analog to induce a continuous headward fluid shift. HDTBR was carried out for 60 days for all subjects. Short-arm centrifugation was utilized to generate AG equating to ~0.3g of acceleration at the level of the eye. The subjects were divided equally into three groups: No AG (control; n=8), daily intermittent AG (6 x 5 min iAG; n=8), and daily continuous 30 min (cAG; n=8). All studies were performed on a single dedicated 3T MRI Scanner. Pulse-gated MRI phase-contrast flow imaging was used to quantify ICA stroke volume and peak-to-peak CSF flow velocity in the mid cerebral aqueduct. 3D-SPGR was acquired for volumetric segmentation of the brain and CSF spaces. MRI acquisitions were obtained at baseline (BDC), 14 days into HDTBR (HDTBR14), 52 days into HDTBR (HDTBR52) and 3-5 days after HDTBR (recovery, R+3/5). The data were analyzed by the mixed model, which included intervention and time (BDC, HDTBR 14, HDTBR 52, R+3/5) as the fixed effects and included subject as the random effect.

RESULTS

24 healthy subject volunteers (16 men, 8 women, mean age = 33 years ± 9 [standard deviation] and mean BMI = 24.3 kg/m2 ± 2.0) successfully completed all phases of the study. Strict six-degree HDTBR was characterized by progressive and statistically significant (p<.01) increases in mean combined brain and CSF volumes and mean aqueductal CSF peak-to-peak flow velocity, as well as statistically significant (p<.01) progressive decrease in mean ICA stroke volume from baseline to 52 days post intervention (Figs. 1-3). Compared to baseline, only combined brain and CSF volumes did not return to baseline values in the recovery period (p=NS). Neither iAG nor cAG exerted any significant effects on the measured MRI brain parameters as compared to HDTBR alone (p=NS).

CONCLUSION

Our results indicate that HDTBR at 6-degrees was effective in producing alterations in ICA stroke volume, aqueductal CSF flow velocity, and combined brain and CSF volumetric change that is associated with chronic headward fluid shift. Short duration, 30-min daily exposure to either iAG or cAG appears to be insufficient in preventing or reducing the effects of chronic HDTBR and thus may not be a suitable countermeasure as currently deployed. AG protocol modifications, including increased duration and magnitude of exposure, should be considered for future research.

REFERENCES

1.	Lee AG, Mader TH, Gibson CR, et al. Spaceflight associated neuro-ocular syndrome (SANS) and the neuro-ophthalmologic effects of microgravity: a review and an update. NPJ Microgravity 2020;6:7. DOI: 10.1038/s41526-020-0097-9.

2.	Kramer LA, Hasan KM, Sargsyan AE, et al. Quantitative MRI volumetry, diffusivity, cerebrovascular flow, and cranial hydrodynamics during head-down tilt and hypercapnia: the SPACECOT study. J Appl Physiol (1985) 2017;122(5):1155-1166. DOI: 10.1152/japplphysiol.00887.2016.