Hello -- thank you for inviting me to speak. My name is Florence Tan. I'm the deputy chief technologist for the Science Mission Directorate. I'm also the chair of the small spacecraft coordination group at NASA.

I will be discussing some exciting SmallSat science missions and how they contribute to protecting and improving life on Earth and in space.

Well, the purpose of this talk, I would like to start off with the definition of a SmallSat --  A spacecraft that is interface compatible with an ESPA ring a dedicated small or medium launch vehicle or containerized dispenser and with an upper mass limit of approximately 500 kilograms.
We also have 1U CubeSats and these are 10 centimeter cubes. 3Us are about the size of a shoebox and 6U are the size of 2 shoe boxes side by side.

Just some history, it turns out NASA deployed small sub satellites in 1971 and 1972 as Apollo 15 and 16 left lunar orbit.
Here you can see the dispenser of this satellites circled in yellow. This was probably the first time that small spacecraft was released into space from a containerized deployment system.

The astronauts you see with one of these sub satellites - it carried 3 instruments, a magnetometer, a charged particle detector, and an S band transponder. It made a number of important measurements of plasma, particle, and the magnetic field environment of the moon as well as mapping the lunar gravity field.

So why SmallSats?

So, this is a picture of of Saturn and is an illustration of the Cassini Mission’s amazing discoveries at almost the end of the mission. This mission is near and dear to my heart. In my 20s, I designed electronics and wrote software for the mass spectrometer instrument on this mission. When it was running low on fuel their mission’s team steered the spacecraft spectacularly close to Saturn in 22 orbits --- this was known as the grand finale --- before the final plunge into Saturn in 2017. They were able to take Risks that they will not able to before, because of the grand finale. The data from the final orbit of Cassini represents a huge leap in our understanding of the Saturn system.

According to the chief scientist Linda Spilker, “Almost everything going into that region turned out to be a surprise. That was the importance of going there – to explore a place we never been before. This expedition really paid off and the data was tremendously exciting”.

Small spacecraft to incredibly attractive compared to flagship missions because of their lower costs and their higher tolerance for risk and with more rapid solicitations. We are able to get more science and faster science.
What's more, constellations of SmallSats enable a path towards new science observations.

SmallSats are able to increase our science capabilities -- these capabilities are real and measurable and the potential is great. SmallSats represents a game changer in the way we achieve science. With SmallSats, we are also able to train the workforce and prove out new technologies.

We've written a document to formalize the overarching integrated strategy and implementation plan approach for SmallSats as a platform. We've identified 4 themes high priority science, support to human exploration, disruptive technology innovation, and regular access to space.

This is our SmallSat CubeSat Fleet chart. The legend on the right shows the color coding. Green means Earth Science Studies Orange Halo Physics and so on.
Missions that are in development, unbolded text and those are launching operating are in non bolded text. We have a total of about 24 missions in operations 72 in development and 32 completed.

This is another way of looking at our missions now, white text denotes missions with the smaller CubeSats. These are the 3U and the 6 U missions. The blue, the green and the orange text denote larger spacecrafts. The yellow text denote missions with 2 or more spacecraft.
Missions to the left of the line represent those in development and missions to the right of the line represent those already launched, deploy, operating, or deorbited. As you can see, in the near future, we will be launching more SmallSats. ESPA class SmallSats, 12 U CubeSats, and constellation missions from the text colors.

Now I'd like to pivot to provide you with a look at our key science themes. All our science missions strived to address some or all our key size themes. Discover the secrets of the universe. Search for life elsewhere and protect and improve life on Earth and in space.

This is the host of satellites that monitor our changing climate and our dynamic universe.

You have seen our fleet chart style missions that we're deploying to study our solar system and our universe. We now turn our attention to our home planet to protect and life on Earth and in space. The next generation of Earth Science begins with this Earth System Observatory we will collect data to study the atmosphere, the ground, and even what's beneath the surface. We will integrate this data in a common approach.

Together will have a 3 D understanding of our earth systems how they work and how one change can influence another. We will watch our planet change and drive solutions for better living, to manage our water and food resources, predict natural hazards, and to cope with sea level rise in our coastal communities, and heat islands in our cities. SmallSats will play an important role to these goals. The next charts, will be a small sampling of the SmallSats we are using at NASA to perform our high value science and towards measurement of our Earth System Observatory.

CYGNSS stands for Cyclone Global Navigation Satellite System. It was launched in 2016 and is a constellation of 8 microsatellites. CYGNSS measures the strength of GPS signals reflected by the earth surface to derive the properties of the surface. CYGNSS was able to make measurements of global ocean surface winds such as tropical cyclones to help understand meteorological processes and improve numerical weather forecasts. You see Pictures of Hurricane Dorian overpass in 2019 and Before and after landfall of Hurricane Isaias last year and CYGNSS also produces daily soil moisture data product, which are used in hydrological process studies and for disaster monitoring.

CYGNSS also developed a new way to spot microplastics in oceans across the globe.

And to track them over time, providing day by day timeline of where they enter the water how they move and where they tend to collect.

We see microplastics in Islands and shore communities around the world as summer approaches, the plastics are in the highest concentration in North Atlantic and Pacific and as summer ends in the Northern Hemisphere and begins in the Southern Hemisphere, microplastics concentration rises in South Pacific, South Atlantic and Indian Ocean.

It's the lowest concentration in January in the winter in the North Hemisphere.

Tempest-D or the temporal experiment for storms and tropical systems. It's a technology demonstration mission to enable millimeter wave radiometer technologies. This mission provided the first ever temporal observations of cloud and precipitation processes and a global scale. It was in orbit for 3 years.

Tempest-D and RAINCube, another CubeSat Mission – RADAR in a Cube mission, deployed from the International Space Station together. These 2 missions allowed NASA to experiment with new techniques for observing weather. This chart shows a 3 D structure of Hurricane Trami. The data from RAINCube -- the vertical data, and the data from Tempest-D -- horizontal data --, combined to show complimentary nature of both missions. This demonstrated that constellations can enable new methods of observation. This is the first heterogeneous consolation measurement. Imagine a future where we have a constellation of Tempest-D and RAINCubes observing weather, following storms as they move to even more detailed forecasts.

TROPICS stands for Time Resolved Observations of Precipitation structure and storm Intensity with a Constellation of SmallSats.

It is a constellation of 6 U CubeSats in 3 orbital planes. It'll launch in 2022 and will fly scanning microwave radiometers that will measure temperature, humidity, precipitation, and cloud properties.

It will offer unprecedented combination of horizontal and vertical temporal resolution to measure environmental and inner core conditions of tropical cyclones at a near global scale. This is a profound leap in temporal resolution in several key parameters needed for the detailed study of high impact meteorological events.

We took advantage of a rideshare opportunity to launch TROPICS Pathfinder, the qualification model, this year and immediately after launching, TROPICS Pathfinder returned the picture of typhoon Mindulle in this chart.

HARP is the Hyper Angular Rainbow Polarimeter. It is 3 U CubeSat launched in 2020 and its measurements will help us understand how clouds and aerosols impact the weather, climate, and the air quality. HARP is was developed initially as an air campaign called ACEPOL. HARP also inspired a successor instrument called HARP 2 and you'll fly on the Planet Aerosol Cloud ocean Ecosystem mission or PACE and with the success of HARP, we will be able to operate HARP all the time and collect significantly more data than the HARP CubeSat.

This is the first light data that HARP CubeSat achieved in April this year.

Temple and Sentinel 4 will join KARI’s GEMS mission to form an international global GEO Constellation to monitor air quality this decade and next focusing on intercontinental pollution transport across the Pacific and Atlantic. On time scales never observed before, we can trace hot spots of high concentration of pollutants. These are ozone and particulate matter. And you can see the pollution from the colors red being the higher concentration. We will feed this data into our models. And for the first time, these missions will provide scientists with a more complete picture of the near global air pollution on an hourly timescale. Pollution knows no borders.

K PLO as you know is the Korea Pathfinder Lunar Orbiter KARI’s Lunar mission scheduled to launch next year. It'll carry a compliment of experiments from South Korea as well as in US built instrument. KPLO’s PolCam, is the precursor of PolCube, a CubeSat that will fly a polarimeter to make targeted observations of the Earth’s aerosols and clouds over land and ocean.

It is an ongoing partnership with a US and South Korea and collaboration between KASI, Busan City, Kyung Hee University, US Army Research Lab and NASA. POLCube UV measurements will compliment GEMS, TEMPO, and Sentinel-4 observations and its measurements can improve the Retrieval and understanding of Aerosol Absorption.

DART is the double asteroid redirection test machine and it is a first demonstration of the Kinetic Impact their technique to change the motion of an asteroid in space. We will crash DART into Dimorphis, a moonlet of the binary asteroid DIdymos, at a speed of approximately 6.6 kilometers/s. This collision will change the speed of the moonlet in its orbit around the main body by a fraction of a percent, but this change will change the orbital period of the moon by several minutes in order to be observed and measured by telescopes on Earth. DART is also carrying LICIACube, which Is a CubeSat on the picture on the left. LiciaCUBE is contributed by the Italian Space Agency. It'll be deployed from DART close to the moonlet before impact and will perform an autonomous flyby of Didymus to monitor the effects of the impact -- the crater, the evolution of the debris and all this data will be very important to verify the efficacy of the impact. DART is a planetary defence-driven test of technologies for preventing and impact of Earth by a hazardous asteroid -- very important in protecting life on Earth, the impact will be late September or Early October, depending on our launch date.

And finally Sunrise. Sunrise is assigned radio interferometer space experiment and we will study the sun 's coronal mass ejection or CME.

This is a CME.

We've known for decades at CME can accelerate particles and produce radio emissions. We are interested in how Solar energetic particles or SEP are accelerated and how they're released into interplanetary space.

Sunrise is a constellation of six 6U CubeSats operating as a radio interferometer to produce a synthetic aperture of approximately a diameter of 10 kilometers; the idea is we take a picture of the CME that's equivalent to this optical picture, using our CubeSats. We will design orbits very carefully so they stay within the 10km sphere. Our CubeSats also pick up GPS signals. And we will have knowledge of where the spacecraft are, we transmit the data to the ground. We post process it. And we could reconstruct this 10 km diameter Space Telescope and figure out where the particles are being accelerated. A key innovation is a combined the radio receiver and the global navigation satellite system receiver to produce precision spacecraft position and a timestamp science data. Sunrise is new in the sense that we're taking a lot of components that have been previously proven out and combining in a novel way to build this 10 kilometers scales. Radio telescope to study the fundamental aspects of physics and particle acceleration for space weather forecast to eventually protect our astronauts earth and space assats.

00:17:49.630 --> 00:18:20.860

Today, we are able to leverage many industry innovations and technologies to enable smallsat missions. The industry has matured reliable SmallSat comm systems, propulsion, and spacecraft buses, etc. We have myriad methods for access to space. We can even buy data to complement our science investigations. These all provide exciting opportunities to expand our science missions as we launch more missions.

We need to be responsible and conscientious users and share the precious resource of the low Earth orbit to adhere to minimum collision avoidance requirements and associated operational and the orbit protocols for our missions.

Looking forward, SmallSat measurements are coming of age. As our associate administrator of science, Thomas Zurbuchen said, “These missions will do big science, but they're also special because they come in small packages, which means we can launch them together and get more research for the price of a single launch”. As our sensors mature, data products from large and small missions become indistinguishable. SmallSats will play an important role in collecting these data. We will gain new insight from multi-instrument constellations data fusion and analytics. And we will need new tools to understand the data. We will take advantage of cooperative synergies among large and small missions. International cooperation is key to our science measurements for the benefit of mankind. And the commercial infrastructure will be leveraged towards this goal as well.

To protect and improve life on Earth and in space will be looking at these processes at Micro Scale, at large convective scales, on land on the oceans, we analyze the data, we will feed the data into our weather models, into our climate models. This will allow us to predict and project the future with far more confidence. Our missions will help farmers grow food around the world predict flood inundation, typhoons, hurricanes and other disasters. Our data will support decadal class observations such the Earth Science Observatory to understand how Earth is changing. We will expand our partnerships with commercial companies and International Space agencies to take advantage of innovation and new technology. We recognize because of the global nature of the issues at hand, we want international partners to be part of this effort --working together, the science we do will serve us all. The climate is changing. We've documented these changes, we've seen them over the last decades.We know it is being driven by human activities. It is absolutely essential that we continue to understand what is happening in order to predict what is going to happen and perhaps Help people make better choices.  Understanding our planet, our planet and its climate Is the foundation for a more resilient and sustainable future. SmallSats will play an important role. We all have a role to play working in cooperation on a mission to the only planet we call home. Thank you very much.