Low Cost Micro-Spectrometer for Resource Mapping on Mars.  S. H. Choi1 and R. W. Moses2; 1Advanced Materials and Processing Branch, Research Directorate, NASA Langley Research Center, MS 188A, Hampton, VA 23681-2199; PH (757) 864-1408; email: sang.h.choi@nasa.gov;  2Atmospheric Flight and Entry Systems Branch, Engineering Directorate, NASA Langley Research Center, MS 489, Hampton, VA 23681-2199, email: robert.w.moses@nasa.gov

Abstract:   Scientific measurements within craters and along steep embankments on Mars pose extreme challenges since astronauts and rovers cannot access them.  However, these areas offer the potential for the greatest scientific return.  NASA Langley Research Center (LaRC) has invented a deployable wireless micro-spectrometer [1-13] capable of accessing those challenging areas and, using telemetry, send measurements to a receiver safely outside those extreme environments.  A prototype of the micro-spectrometer concept was fabricated and successfully tested for measurement of dopamine as a neural probe [14].  The bullet-sized micro-spectrometer concept can be designed for deployment by an astronaut, a rover, an aerial drone, or a lander, and the measurements broadcasted back to a receiver [13,15,16].  This device can also be installed on rover tires, under the astronaut’s boots, or a cane stick. The bullet-like consumable micro-spectrometer can penetrate soil to spectrally identify the components of the soil, such as water or minerals. The signals of the soil assay data are transmitted to a mother station through a telemetry system. The LaRC developed micro-spectrometer bullet consists of micro-spectrometer optics with an all imbedded burst-mode LED UV light source, a super-capacitor with control electronics, and telemetry electronics.  Prototypes have been fabricated to demonstrate spectral assay of soil components. Further maturation of this technology would be necessary for demonstration on Mars.  Future work includes a potential R&D program that includes other NASA Centers and industry partners interested in developing a business case that requires knowledge of the locations of extraterrestrial ice, for instance.

REFERENCES:

- Park, Y. and Choi, S.H. (2010), “Linear Fresnel Spectrometer Chip with Gradient Line Grating”, NASA Case No. LAR 17947-1, August 17, 2010.

- U.S. Patent 9,046,418 B1, June 2, 2015.

- Park, Y., Wright, J.D., Jensen, J.D.L., King, G.C., Choi, S.H. (2005), “Diffraction Analysis for Periodic Nano-scale Apertures, Scatterers and Absorbers”, IOP Journal, Measurement Science and Technology, 16, pp2208-2212.

- Park, Y., Koch, L., Park, S.J., King, G.C., Song, K.D., Choi, S.H. (2008), “Miniaturization of a Fresnel Spectrometer”, Journal of Optics A: Pure and Applied Optics, Vol. 10, 095301, doi: 10.1088/1464-4258/10/9/095301.

- Park, Y. and Choi, S.H. (2013), “Miniaturization of Optical Spectroscopes into Fresnel Micro Spectrometer”, Commemorative Paper, Journal of Nanophotonics, Vol. 7, 077599.

- U.S. Patent 8,089,677, January 3, 2012.

- U.S. Patent 7,379,231 B2, May 27, 2008.

- U.S. Patent 8,294,989, October 23, 2012.

- U.S. Patent 8,015,815 B2, September 13, 2011.

- U.S. Patent 8,174,695, May 8, 2012.

- U.S. Patent 8,059,273 B2, November 15, 2011.

- U.S. Patent 8,094,306, January 10, 2012.

- Sang H. Choi and Robert W. Moses (2019), NASA Invention Disclosure, “Deployable Micro-spectrometer Bullets”, NASA Case No. LAR 19610-1, e-NTR #: 1552997528, March 19, 2019.

- Min Hyuck Kim, Hargsoon Yoon, Sang H. Choi, Fei Zhao, Jongsung Kim, Kyo D. Song, and Uhn Lee, “Miniaturized and Wireless Optical Neurotransmitter Sensor for Real-time Monitoring of Dopamine in the Brain”, Sensors, Vol. 16, Issue 11, (ISSN 1424-8220; CODEN: SENSC9), Nov. 2016.

- Sang H. Choi and Robert W. Moses, “Micro-Spectrometer for Resource Mapping in Extreme Environments”, IAC-19- D1.3.9x52141, Proceedings of 70th International Astronautical Congress (IAC), Washington D.C., 21-25 October 2019.

- Sang H. Choi and Robert W. Moses, “Development of Resource Mapping Device for Extreme Environments”, Proceedings of ASCE Earth & Space Conference, Seattle WA, April 19-23, 2021.