Slide 1: (1 - 17)

Hello, my name is James Akers, and I am from the

NASA Glenn Research Center in Cleveland Ohio,

and I will be presenting paper number 12648,

Challenges and Considerations When Using Hydraulic

Modal Shaking in Large-Scale Modal Testing.

The coauthors are Michael Hale from the Redstone Test

Center and Joel Sills from NASA Johnson Space Center.

In summary the presentation covers some of the unique

challenges when using hydraulic modal shakers, such as

their nonlinear distortion in their shaker force, care in

setting up the hydraulic power supply and hydraulic hosing,

shaker performance limits and the potential for damaging

hydraulic shakers, and safety issues related to operating

high-pressure hydraulic systems.

Recognizing and properly dealing with these challenges

will allow the modal test engineer to successfully utilize

hydraulic modal shakers on large-scale modal tests.

Slide 2: (18 - 29)

Modal tests have traditionally utilized electrodynamic

modal shakers.  They have good linearity characteristics,

are versatile, and relatively easy to setup.

The figures on the right show 50 lbf, 110 lbf, and 250 lbf

electrodynamic modal shakers being used on medium

and medium-large scale modal tests.

However, utilizing electrodynamic modal shakers on

dimensionally larger and more massive test articles,

combined with the need for higher excitation levels in

order to identify their nonlinear characteristics can be

problematic due to longer stroke, higher velocity, and

higher force level requirements.

Slide 3: (30 - 43)

Hydraulic modal shakers become attractive since they

can generate higher force levels at lower unit cost with

a smaller spatial footprint and can better meet the longer

stroke, higher velocity, and higher force requirements.

Four fixed hydraulic modal shakers were used in modal

testing NASA’s 327 feet tall and 1.8 million pound Ares

I-X Flight Test Vehicle (FTV) inside the Vehicle Assembly

Building (VAB) in 2009, shown in the bottom left.

More recently in June 2019, five inertial hydraulic modal

shakers, two vertical and three horizontal shown in the

bottom right, were used in modal testing the Space

Launch System (SLS) Mobile Launcher (ML), which is

over 360 feet tall and weighs more than 10 million

pounds, inside the VAB and is shown in the bottom center.

Slide 4: (44 - 68)

Hydraulic actuators are comprised of a piston in a

housing with pressurized hydraulic fluid used to exert

a pressure to the face of the piston to generate motion

and/or force.

Hydraulic actuators used for vibration and shock testing

use a stiff double acting double ended (i.e., double rod)

hydraulic actuator with a high frequency response

electrohydraulic servo valve.

Push/pull forces and/or extension/retraction of the actuator

is achieved by changing the pressure differential across

the piston.

The figure on the bottom left shows the double acting

double ended hydraulic actuator configuration.

For the double acting and double acting double ended

hydraulic actuator configurations, two hydraulic fluid

hoses run between the hydraulic power supply and the

hydraulic actuator with the feed line supplying the higher

pressure hydraulic fluid and the lower pressure return

line bringing back hydraulic fluid from the hydraulic

actuator to the hydraulic power supply reservoir.

The figure on the bottom right shows how the motion of

the servo valve sends high pressure source hydraulic

fluid to and pulls the lower pressure return hydraulic

fluid from a double acting hydraulic actuator in order to

make it extend and retract.

Slide 5: (69 - 94)

A servo controller sends an electrical command signal

to the electrohydraulic servo valve that positions the

servo valve and therefore the piston at the desired

starting position (usually mid-stroke).

The servo controller uses position feedback of the servo

valve and the hydraulic actuator piston, usually measured

with a Linear Variable Displacement Transducer (LVDT).

Hydraulic actuators are inherently unstable and the servo

controller is required to set the desired hydraulic actuator

piston position.

Without the servo controller, the hydraulic actuator piston

would end up drifting off to one side or the other reducing

the useful stroke limit and potentially impacting its

mechanical stops and incurring damage.

The servo controller, its electrical command signal to

the electrohydraulic servo valve, and the hydraulic

actuator position and servo valve position signal feedback

is often referred to as the “inner control loop”.

The “inner control loop” does not prevent the hydraulic

actuator from reaching its stroke limit.

Depending upon how hard the hydraulic actuator is

driven an “outer control loop” either open-loop or

closed-loop may be required to prevent the hydraulic

actuator from reaching its stroke limit.  The “outer control

loop” is generally based on force or acceleration

feedback.

Slide 6: (95 - 123)

The servo controller in addition to generating the electrical

command signal for the servo valve to move the hydraulic

actuator piston, based upon the shaker controller’s drive

signal, also generates a high frequency dither signal.

The dither signal produces very small amplitude

Oscillations or dithering in the servo valve to prevent it

from sticking, which if this occurred could end up

producing uncommanded high amplitude hydraulic

actuator motion or forces that could potentially damage

the test article.

Dither signal frequency should be well above the modal

test frequency range of interest, typically an octave or

more above the resonant frequency of the servo valve

resonant frequency where the system transfer function

has minimal gain.

The dither signal may produce local responses in the

test article, particularly in the vicinity of the shaker

drive points, and therefore may affect the data acquisition

system (DAQ) full scale settings.

Recommend at least one set of data be acquired at a

sampling frequency high enough to capture the dither

signal and the test article’s response to it while the shaker

drive signal to the servo controller is off.

The figure on the right shows the Power Spectral

Density (PSD) of a hydraulic modal shaker force.  The

dither signal frequency is more than a decade above

the frequency range of interest and has a magnitude

that is six orders of magnitude less than the shaker force

over the frequency range of interest.

Slide 7: (124 - 131)

Hydraulic actuators have a dynamic and a mechanical

stroke limit.

When the hydraulic actuator reaches it dynamic stroke

limit, the piston starts to engage its dashpots that begin

to bleed off the hydraulic fluid that is applying the force

on the piston in order to try and reduce the piston’s

velocity prior to reaching its mechanical stroke limit

where it impacts the housing’s mechanical stops.

Hydraulic actuators should not be driven to exceed

their dynamic stroke limits for two reasons:

First, engaging the dashpots introduces more nonlinear

distortion in the hydraulic actuator force.

Second, the distance between the dynamic and

mechanical stroke limits is relatively small, so exceeding

the dynamic stroke limit likely means the mechanical

stroke limit will be reached.

Repeated high velocity contact with the mechanical

stops can result in permanent damage.

Slide 8: (132 – 156)

Hydraulic modal shaker forces can have significant

nonlinear distortion.

This nonlinear distortion is primarily due to the servo

valve having a nonlinear flow response, which is a

function of the area of the control orifice and the square

root of the pressure drop across the orifice.

Compounding this nonlinear effect is the hydraulic

actuator acts as a lightly damped single-degree-of-

freedom (SDOF) oscillator that amplifies pressure

oscillations close to its oil column resonance frequency.

The equation for the oil column resonance frequency

for a double acting double ended hydraulic actuator is

shown below on the left.

The effective bulk modulus of the hydraulic fluid is a

function of pressure and the amount of entrained air in

the hydraulic fluid.  The more entrained air, the lower

the effective bulk modulus.

Entrained air can also lead to the formation of air

bubbles in the hydraulic fluid that can result in cavitation

in the hydraulic pumps and cause their failure.

The figures at the bottom center show an example of

the nonlinear distortion in a hydraulic modal shaker force

during a linear sine sweep and the corresponding PSD

waterfall plot shown on the bottom right clearly shows

the dominance of the odd superharmonics.

Slide 9 (157 – 217)

Hydraulic modal shakers performance limits are defined

by its peak displacement, peak velocity, peak dynamic

force, oil column resonance frequency, and servo valve

corner frequency.

The hydraulic modal shaker’s oil column acts as a

second order mechanical system that above the oil

column resonance frequency has a slope proportional

to the inverse of the frequency squared (i.e., -40 dB/dec

or -12 dB/oct).

The servo valve acts as a first order mechanical system

that above its corner frequency has a slope proportional

to the inverse of the frequency (i.e., -20 dB/dec or -6 dB/oct).

Hydraulic modal shakers displacement limits are typically

set to be equal to or slightly less than their dynamic stroke

limit in order to avoid engaging the dashpots or

accidentally reaching their mechanical stroke limit.

Hydraulic power supplies are designed to produce a

specified flow rate of hydraulic fluid.  The pressurization

that results is due to the resistance to that flow.

Therefore, as the hydraulic modal shaker actuator piston

velocity increases the hydraulic pressure decreases.

Hence the hydraulic modal shakers mid frequency

velocity limit is a function of both the hydraulic power

supply maximum flow rate and the minimum pressure

needed to adequately “float” the hydraulic actuator piston

in order to prevent it from rubbing up against the housing

and undergoing scoring.

The plateau in the high frequency acceleration limit is a

function of the peak dynamic force the hydraulic modal

shaker can produce and the total amount of mass it is

driving, which includes the actuator piston mass.

At frequencies above the oil column resonance frequency

and servo valve corner frequency, the high frequency

acceleration limit curve has a slope proportional to the

inverse of the frequency cubed (i.e., -60 dB/dec or -18 dB/oct).

The figure on the top right shows an example of a

theoretical hydraulic modal shaker performance limit

curves for a fixed shaker with a peak force of 100 lbf and

assumes no nonlinear distortion.

The solid black line is the performance curve when the

shaker is pushing on 50 lbm, the oil column resonance

frequency is 70 Hz, and the servo valve corner frequency

is 150 Hz.

Note that when this shaker is pushing on a 100 lbm, the

acceleration limit is lowered sufficiently that the hydraulic

modal shaker is not velocity limited.

Nonlinear distortion in the hydraulic modal shaker force

may result in the theoretical shaker performance limit

curve not being a conservative upper bound.

Therefore, an accurate shaker performance limit

curve may need to be empirically derived.

The figure in the bottom right shows an actual performance

limit curve for an inertial hydraulic modal shaker pushing

on a fixed point derived from sine sweep test data that

was sine post processed using a bandpass tracking filter

to determine the amplitude of the fundamental harmonic.

The bucket in the knee and the knee itself are a result

of the velocity limit (i.e., need for maintaining a minimum

hydraulic pressure).

The oil column resonance frequency and servo controller

corner frequency are above the frequency range of

interest resulting in no high frequency rolloff in this plot.

Slide 10 (218 – 244)

When using hydraulic modal shakers and their associated

hydraulic power supplies and hydraulic hosing, safety

procedures should be followed to protect personnel, test

article, and facility.

Potentially high force transient events can be caused by

a major failure in the servo controller or in the feedback

signals.

This failure mode is very similar to the failure mode

produced by a major failure in the electrodynamic modal

shaker amplifier (e.g., op amp fails).

Loss of the command signal to the servo controller will

cause the hydraulic modal shaker to use all its available

force to remain stationary and therefore fix the drive point,

which can produce high loads into the test article.

Personnel should not be touching or near a hydraulic

modal shaker when the servo controller is on regardless

of whether the hydraulic power supply is on because

there can be significant stored energy in the hydraulic

system.

This failure mode also needs to be considered when

sizing the hydraulic modal shakers, in the selection of

the shaker attachment hardware, and in selecting the

drive points.

Uninterrupted Power Supply (UPS) backups to the servo

controllers are highly recommended to ensure the servo

controllers remain under power in the event of a power

outage to minimize any transient forces into the test article.

Slide 11 (245 – 270)

Of fundamental importance is that the pressure rating

of the actuators, bearings, and supply lines are

documented and that the hydraulic power supply is not

allowed to exceed these maximum pressure limits.

Safety issues related to spillage and leakage need to

be considered in the test planning, preparation, and

execution.

A drip pan to collect dripping/leaking hydraulic fluid

from the hydraulic actuator and its hydraulic fittings

may be needed.

Some amount of residual leakage is to be expected

and keeping the work area associated with any servo-

hydraulic system clean should be considered mandatory

continual maintenance.

Safety procedures for inspection and detection of leaking/

cracked hydraulic hoses should be implemented to

prevent potential contamination of the test article with

hydraulic fluid.

Personnel safety procedures should be followed to

prevent personnel from being exposed to the hazards

from operating hydraulic systems:

Sprayed by the hot hydraulic fluid.

Hit by flailing hydraulic hoses.

When handling hoses the potential of having hydraulic

fluid injected under the skin, which can have grave

medical complications.

Slide 12 (271 – 304)

The hydraulic power supply consists of a hydraulic fluid

reservoir tank, hydraulic pump, heat exchanger to maintain

the hydraulic fluid temperature, and filtration to maintain

the cleanliness of the hydraulic fluid.

It is estimated that at least 75% of hydraulic system

failures and maintenance activities are the result of

contaminated hydraulic fluid.

Proper filtration and temperature control are essential

for maintaining the quality of the hydraulic fluid and the

health of the hydraulic actuator.

Hydraulic fluid cleanliness should be tested on a regular

basis based on system usage and any time there is a

major component failure such as a damaged bearing or

valve.

When the system is idle, it is highly recommended that

hydraulic fluid be continually circulated through the

filtration system and that a near operational temperature

be maintained.

The hydraulic power supply heat exchanger needs to be

capable of properly maintaining the temperature of the

hydraulic fluid.

Hydraulic fluid contamination due to the introduction of

water must be prevented.

This paper has covered the basics of electrohydraulic

servo valves, hydraulic actuators, and the safety issues

and best practices that should be followed when

operating and maintaining a high pressure hydraulic

system.

Keeping these things in mind, the test engineer can find

hydraulic modal shakers to be a valuable tool in

performing high force large scale modal tests.

The authors would like to thank the NASA Engineering

Safety Center (NESC), NASA Glenn Research Center,

and the Redstone Test Center for funding this work.