- Supporting Information —

Developing a Strategy for the National Coordinated Soil Moisture Monitoring Network

Michael H. Cosh1, Todd Caldwell2, Bruce Baker3, John D. Bolten4, Nathan Edwards5, Peter Goble6, Heather Hofman7, Tyson E. Ochsner8, Steven Quiring9, Charles Schalk10, Marina Skumanich11, Mark Svoboda12, and Molly Woloszyn11

1 U.S. Department of Agriculture, Agricultural Research Service, Beltsville Agricultural Research Center, Hydrology and Remote Sensing laboratory, 10300 Baltimore Ave, Beltsville, MD 20705

2 U.S. Geological Survey, Nevada Water Science Center, 2730 N. Deer Run Rd., Carson City, NV 89701

3 National Oceanic and Atmospheric Administration, Air Resources Laboratory, 456 South Illinois Avenue

Oak Ridge, TN  37830

4 Hydrological Sciences Laboratory, Code 617, NASA Goddard Space Flight Center, Greenbelt , MD 20771

5 South Dakota State University, 13 2nd Ave SE, Aberdeen, SD 57401

6  Colorado State University, Colorado Climate Center, 1371 Campus Delivery, Fort Collins, CO 80523

7  U.S. Department of Agriculture, NRCS National Water and Climate Center, 1201 NE Lloyd Blvd, Suite 802 Portland, OR 97232

8 Oklahoma State University, Department of Plant and Soil Sciences, 371 Agricultural Hall, Stillwater, OK 74078

9 Ohio State University, Department of Geography, 154 North Oval Mall, Columbus, OH 4321

10 U.S. Geological Survey, Hydrologic Networks Branch, 196 Whitten Road, Augusta, ME 04330

11 National Oceanic and Atmospheric Administration, National Integrated Drought Information System,

12 University of Nebraska-Lincoln, National Drought Mitigation Center, 3310 Holdrege Street, Lincoln, NE 68583

Supporting information includes references for Table 1 and Table 2.

Table 1. A summary of common (but not all inclusive) in situ and profile sensor errors, as root mean squared error (RMSE), stated from the manufacturer and determined by researchers using the factory standard coefficients and soil-specific calibrations.

| In situ Sensors |  |  | Freq. (MHz) |  | RMSE (m3/m3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Company | Type1 |  | Output2 | Stated | Standard calibration | Soil specific | Reference |
| 10HS | Meter | Cap. | 70 | V | 0.03 | 0.073, 0.053 | 0.013, 0.012 | [1], [2] |
| 5TEd | Decagon | Cap. | 70 | Ka, EC, T | 0.03 | 0.040, 0.039 | 0.026, 0.013 | [1], [3] |
| CS616 | CSI | TLO | 175 | period | 0.025 | 0.057,0.129, 0.073 | -, 0.025, 0.063 | [4], [1], [5] |
|  |  |  |  |  |  | 0.140, 0.157 | 0.027, 0.016 | [6], [3] |
| CS650/655 | CSI | TLO | 175 | Ka, EC, T | 0.03 | 0.073, 0.078 | 0.025, 0.022 | [7], [3] |
| Digital TDT | Acclima | TDT | 1230 | Ka, EC, T | 0.02 | 0.049, 0.080 | -, 0.025 | [4], [5] |
| EC-5d | Decagon | Cap. | 70 | V | 0.03 | -, 0.054 | 0.013, 0.025 | [8], [3] |
| Field Connect | J. Deere | Cap. |  |  |  | 0.083 | 0.026 | [3] |
| Hydra Probe | Stevens | Imp. | 50 | Ka, EC, T | 0.01 | 0.073, 0.033, 0.048 | 0.056, 0.022, 0.028 | [9], [10], [1] |
|  |  |  |  |  |  | 0.040, 0.102, 0.010 | 0.029, 0.013, - | [5], [3], [11] |
| SM150/300 | Delta-T | Imp. | 100 | V, T | 0.03 | 0.037 | 0.014 | [1] |
| TDR100d/ TDR200 | Campbell | TDR | 1450 | Ka, EC | - | 0.042, 0.023 | -, 0.022 | [4], [1] |
| TDR315 | Acclima | TDR |  |  | - | 0.050, 0.020 | 0.016, - | [3], [11] |
| Theta Probe | Delta-T | Imp. | 100 | V | 0.01 | 0.066, 0.029, 0.030 | -, 0.015, 0.028 | [4], [1], [5] |
| Trime-PICO | IMKO | TDR | 1000 | V | - | 0.042, - | 0.023, 0.044 | [5], [12] |
| Wet | Delta-T | Cap. | 20 | Ka, EC, T | 0.03 | 0.041, 0.034 | 0.029, 0.025 | [13], [1] |
| Profile Sensors |  |  |  |  |  |  |  |  |
| AquaCheck | – | Cap. |  |  | - | 0.163 | 0.013 | [3] |
| Diviner 2000 | Sentek | Cap. | 250 | counts | - | 0.030–0.053, - | 0.025, 0.018-0.044 | [14], [15] |
| EasyAg | Sentek | Cap. |  | – | 0.06 | - | - |  |
| EnviroSCAN | Sentek | Cap. | 75 | count |  | 0.018 – 0.073, - | 0.020, 0.021-0.051 | [14], [15] |
| Gro-Point | ESI | TDT |  | current |  |  |  |  |
| PR2/6 | Delta-T | Cap. | 100 | V | 0.04 | 0.091–1.30, - | 0.027, 0.024–0.063 | [14], [15] |
| SoilVUE-10 | Campbell | TDR | 1450 | Ka, EC, T | 0.02 |  |  |  |
| Trime-T3 | IMKO | TDR |  | time (ps) | 0.03 | 0.051- 070 | 0.020 | [14] |

dDiscontinued sensor, - indicates no value stated in reference.

1Sensor type: Cap., capacitance; Imp., impedance; TLO, transmission line oscillator; TDR, time domain reflectometry.

2Sensor outputs includes Ka, dielectric permittivity; EC, electrical conductivity; T, temperature; V, analog voltage; time in picoseconds; and periods or pulse counts.

Table 2. The soil moisture products derived from space-borne platforms and operational1 land surface models. References are available in supplemental information.

| Satellite Soil Moisture Missions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mission | Duration | Coverage | Revisit time | Band | Spatial Resolution | Reference |
| AMSR-E (JAXA) | 2002-2011 | Global | 1 day | X/C | 10-50 km | [16] |
| Aquarius | 2011-2015 | Global | 8 days | L | 100 km | [17] |
| ASCAT | 2009-pres. | Global | 2-3 days | C | 25 km | [18] |
| CYGNSS | 2017-pres. | Mid-latitudes | Week-Month | L | 1-3 km | [19, 20] |
| GCOM-W (AMSR2) | 2012-pres. | Global | 2-3 days | X/S | 25 km | [21] |
| Grace/Grace-FO | 2002-pres. | Global | 30 days | K-band Ranging | 200 km | [22] |
| NISAR | 202?-? | Global | 12 days | L/S | 200 m | [23] |
| Sentinel-1 (ESA) | 2015-pres, 2015-pres, | Europe Global index | 3-8 days 1 day | C C | 1 km 0.1 degree | [24] |
| SMAP (NASA) | 2015-pres. | Global | 2-3 days | L | 3 km/9 km/36 km | [25, 26] |
| SMOS (ESA) | 2009-pres. | Global | 2-3 days | L | 25 km | [27, 28] |
| WindSat (DoD) | 2003-2020? | Global | 8 days | X | 25 km | [29] |
| Operational Land Surface Models |  |  |  |  |  |  |
| Product | Models | Coverage | Time | Agency | Spatial Resolution | Reference |
| NLDAS-2 | Noah, Mosaic, SAC, VIC | CONUS | 1979 - pres. | NASA | 0.125 degree (~15 km) | [30] |
| WLDAS | Noah-MP | Western US | 1979 - pres. | NASA | 0.01 degree (~1 km) | [31] |
| National Water Model | WRF-Hydro | CONUS | Short, med, long forecasts | NOAA | 1 km and 250 m | [32] |
| National Hydrologic Model | PRMS | CONUS | 1980 - pres. | USGS | 1 km | [33] |

1Operational implies continuous simulations in near-real time for use operationally by a number of federal services like flood forecasting, drought mitigation, and weather forecasting.

# References

[1]	C. M. P. Vaz, S. Jones, M. Meding, and M. Tuller (2013), Evaluation of standard calibration functions for eight electromagnetic soil moisture sensors, Vadose Zone J, 12, doi: 10.2136/vzj2012.0160.

[2]	J. M. Dominguez-Nino, H. R. Bogena, J. A. Huisman, B. Schilling, and J. Casadesus (2019), On the Accuracy of Factory-Calibrated Low-Cost Soil Water Content Sensors, Sensors, 19, doi: 10.3390/s19143101.

[3]	J. Singh et al. (2018), Performance assessment of factory and field calibrations for electromagnetic sensors in a loam soil, Agr Water Manage, 196, 87-98, doi: 10.1016/j.agwat.2017.10.020.

[4]	J. M. Blonquist, S. B. Jones, and D. A. Robinson (2005), A time domain transmission sensor with TDR performance characteristics, Journal of Hydrology, 314, 235-245.

[5]	M. H. Cosh et al. (2016), The Soil Moisture Active Passive Marena, Oklahoma, In Situ Sensor Testbed (SMAP-MOISST): Testbed design and evaluation of in situ sensors, Vadose Zone J, 15, doi: 10.2136/vzj2015.09.0122.

[6]	C. Rudiger, A. W. Western, J. P. Walker, A. B. Smith, J. D. Kalma, and G. R. Willgoose (2010), Towards a general equation for frequency domain reflectometers, Journal of Hydrology, 383, 319-329.

[7]	T. G. Caldwell, T. Bongiovanni, M. H. Cosh, C. Halley, and M. H. Young (2018), Field and Laboratory Evaluation of the CS655 Soil Water Content Sensor, Vadose Zone J, 17, 170214, doi: 10.2136/vzj2017.12.0214.

[8]	H. R. Bogena, J. A. Huisman, C. Oberdorster, and H. Vereecken (2007), Evaluation of a low-cost soil water content sensor for wireless network applications, Journal of Hydrology, 344, 32-42.

[9]	D. D. Bosch (2004), Comparison of capacitance-based soil water probes in coastal plain soils, Vadose Zone J, 3, 1380-1389.

[10]	M. S. Seyfried, L. E. Grant, E. Du, and K. Humes (2005), Dielectric loss and calibration of the hydra probe soil water sensor, Vadose Zone J, 4, 1070-1079, doi: 10.2136/vzj2004.0148.

[11]	T. B. Wilson et al. (2020), Evaluating time domain reflectometry and coaxial impedance sensors for soil observations by the U.S. Climate Reference Network, Vadose Zone J, 19, 10.1002/vzj2.20013.

[12]	U. Dettmann and M. Bechtold (2018), Evaluating Commercial Moisture Probes in Reference Solutions Covering Mineral to Peat Soil Conditions, Vadose Zone J, 17, 170208, doi: 10.2136/vzj2017.12.0208.

[13]	G. Kargas, P. Kerkides, M. Seyfried, and A. Sgoumbopoulou (2011), WET Sensor Performance in Organic and Inorganic Media with Heterogeneous Moisture Distribution, Soil Sci Soc Am J, 75, 1244-1252, doi: 10.2136/sssaj2010.0238.

[14]	S. R. Evett, J. A. Tolk, and T. A. Howell (2006), Soil profile water content determination: Sensor accuracy, axial response, calibration, temperature dependence, and precision, Vadose Zone J, 5, 894-907, doi: 10.2136/vzj2005.0149.

[15]	N. T. Mazahrih, N. Katbeh-Bader, S. R. Evett, J. E. Ayars, and T. J. Trout (2008), Field calibration accuracy and utility of four down-hole water content sensors, Vadose Zone J, 7, 992-1000, doi: 10.2136/vzj2008.0001.

[16]	E. G. Njoku, T. J. Jackson, V. Lakshmi, T. K. Chan, and S. V. Nghiem (2003), Soil moisture retrieval from AMSR-E, IEEE Transactions on Geoscience and Remote Sensing, 41, 215-229, 10.1109/tgrs.2002.808243.

[17]	R. Bindlish, T. Jackson, M. Cosh, T. J. Zhao, and P. O'Neill (2015), Global soil moisture from the Aquarius/SAC-D Satellite: Description and initial assessment, Ieee Geosci Remote S, 12, 923-927, 10.1109/Lgrs.2014.2364151.

[18]	Z. Bartalis et al. (2007), Initial soil moisture retrievals from the METOP-A Advanced Scatterometer (ASCAT), Geophys Res Lett, 34, L20401, doi: 10.1029/2007gl031088.

[19]	C. C. Chew and E. E. Small (2018), Soil Moisture Sensing Using Spaceborne GNSS Reflections: Comparison of CYGNSS Reflectivity to SMAP Soil Moisture, Geophys Res Lett, 45, 4049-4057.

[20]	H. Kim and V. Lakshmi (2018), Use of Cyclone Global Navigation Satellite System (CyGNSS) Observations for Estimation of Soil Moisture, Geophys Res Lett, 45, 8272-8282, doi:10.1029/2018GL078923.

[21]	R. M. Parinussa, T. R. H. Holmes, N. Wanders, W. A. Dorigo, and R. A. M. de Jeu (2015), A preliminary study toward consistent soil moisture from AMSR2, Journal of Hydrometeorology, 16, 932-947, 10.1175/Jhm-D-13-0200.1.

[22]	B. D. Tapley, S. Bettadpur, J. C. Ries, P. F. Thompson, and M. M. Watkins (2004), GRACE measurements of mass variability in the Earth system, Science, 305, 503-505, 10.1126/science.1099192.

[23]	NISAR (2018), NASA-ISRO SAR (NISAR) Mission Science Users’ Handbook, NASA Jet Propulsion Laboratory, CL# 18-1893, 261 pp.

[24]	F. Mattia et al. (2017), Sentinel-1 high resolution soil moisture, in 2017 IEEE International Geoscience and Remote Sensing Symposium, edited by, p.^pp. 5533-5536.

[25]	S. K. Chan et al. (2018), Development and assessment of the SMAP enhanced passive soil moisture product, Remote Sensing of Environment, 204, 931-941, doi: 10.1016/j.rse.2017.08.025.

[26]	S. K. Chan et al. (2016), Assessment of the SMAP passive soil moisture product, IEEE Transactions on Geoscience and Remote Sensing, 54, 4994-5007, doi:10.1109/Tgrs.2016.2561938.

[27]	Y. H. Kerr et al. (2010), The SMOS Mission: New Tool for Monitoring Key Elements of the Global Water Cycle, Proc. IEEE, 98, 666-687.

[28]	Y. H. Kerr, P. Waldteufel, J. P. Wigneron, J. M. Martinuzzi, J. Font, and M. Berger (2001), Soil moisture retrieval from space: The Soil Moisture and Ocean Salinity (SMOS) mission, IEEE Transactions on Geoscience and Remote Sensing, 39, 1729-1735, 10.1109/36.942551.

[29]	L. Li et al. (2010), WindSat Global Soil Moisture Retrieval and Validation, IEEE Transactions on Geoscience and Remote Sensing, 48, 2224-2241, 10.1109/Tgrs.2009.2037749.

[30]	Y. Xia et al. (2014), Evaluation of multi-model simulated soil moisture in NLDAS-2, Journal of Hydrology, 512, 107-125.

[31]	J. M. Erlingis et al. (2021), A high‐resolution land data assimilation system optimized for the western United States, Journal of the American Water Resources Association, n/a, doi: 10.1111/1752-1688.12910.

[32]	S. Cohen, S. Praskievicz, and D. R. Maidment (2018), Featured Collection Introduction: National Water Model, Journal of the American Water Resources Association, 54, 767-769, 10.1111/1752-1688.12664.

[33]	R. S. Regan et al. (2019), The US Geological Survey National Hydrologic Model infrastructure: Rationale, description, and application of a watershed-scale model for the conterminous United States, Environmental Modelling & Software, 111, 192-203, 10.1016/j.envsoft.2018.09.023.