Cryobotics: Extreme Cold Environment Testing

The extreme cold environment test chamber was designed to conduct research in cryobotics; an area of study that focuses on robotic systems and rotating machinery operating in extreme cold environments including Earth, low Earth orbit, Mars, Moon, asteroids, Solar orbit, planetary orbit, or those encountered during travel among these destinations. The test chamber incorporates a modular dynamometer, consisting of a variety of brakes, torque sensors and motors to be easily interchanged between tests. Each test employs a unique test profile that incorporates different setpoints of applied torques and velocities for a given period or number of revolutions. The modularity of the dynamometer setup allows for any combination of motor, gearbox to be tested.   The chamber has been used to run tests for various projects including Bulk Metallic Glass Gears (BMGG), Volatiles Investigating Polar Exploration Rover (VIPER), Intelligent Payload Experiment (IPEX), and Pilot Excavator.

Various upgrades have been made to the extreme cold environment test chamber for the use of cryobotic research. These upgrades greatly increased the autonomous capabilities of the test set up by providing redundancies in the hardware and software. The redundancies were primarily added to protect the integrity of the cryohead. A new strapping and insulation method was performed to create the thermal conductive path from the actuators to the cryohead. The software was upgraded to include temperature setpoint control, further increasing the autonomous capabilities of the test.

This paper goes into detail regarding the upgrades made to the extreme cold environment test chamber, as well as highlights the results from a COLDArm acceptance test.