Conformal PICA TPS – Enabling Future NASA Planetary Science Missions

Mairead Stackpoole,

Thermal Protection Materials Branch	,

NASA Ames Research Center, Moffett Field, CA

Initial development of conformal PICA (C-PICA) ablative TPS occurred under NASA’s Hypersonics Project in the 2000’s and demonstrated very low through the thickness conductivity compared to state-of-the-art PICA. PICA, which was first demonstrated on Stardust, has some inherent limitations that C-PICA improves on, primarily strain to failure. More recently C-PICA has been further matured and a family of C-PICA materials are now ready for consideration as an enabling technology for New Frontiers and other NASA missions. C-PICA has several improvements compared to PICA including:

- Higher strain to failure and lower thermal conductivity (up to 55% less than PICA depending on C-PICA variant)

- CTE comparable to typical composite carrier structures

- Temperature independent mechanical properties

- Suited for single piece (up to ~ 1.5m) or tiled configurations

- Larger tiles leading to reduced integration complexity compared to tiled PICA

- Reduced mass compared to PICA due to reduced thermal conductivity

C-PICA has been tested at heating fluxes ranging from 250-1850 W/cm2 with excellent performance.

C-PICA model during/post arc jet testing

Based on our evaluation, multiple missions listed on the SMD Technology Showcase will potentially need a thermal protection system capable of withstanding entry environments where C-PICA is suited either as a forebody or backshell TPS. We will be highlighting mature conformal PICA variants to support the following missions at the showcase in January 2023.

| Outer Planets |  |  |
| --- | --- | --- |
|  | Mission Abstract | Comment/Application |
| 1 | New Frontiers Titan Orbiter | Aerocapture requires a mass efficient TPS suite - Conformal PICA is very suitable |
| 2 | Small Next-Generation Atmospheric Probe (SNAP) For Ice Giant Missions | Conformal PICA for backshell TPS |
| 3 | Uranus Orbiter and Probe (UOP) | Conformal PICA for backshell TPS |
| 4 | Saturn Probe | Conformal PICA for backshell TPS |
| Venus |  |  |
|  | Mission Abstract | Comment/Application |
| 5 | SAEVe: Seismic and Atmospheric Exploration of Venus | Conformal PICA for backshell TPS |
| 6 | V-BOSS: Venus Bridge Orbiter and Surface System | Conformal PICA for backshell TPS |
| 7 | Venus In Situ Explorer (VISE) | Conformal PICA for backshell TPS |
| 8 | Cupid’s Boomerang | Conformal PICA for backshell TPS |
| Small Body Sample Return |  |  |
|  | Mission Abstract | Comment/Application |
| 9 | Ceres Sample Return | Conformal PICA is an alternate to PICA forebody TPS with improved mass efficiency |
| 10 | Comet surface sample return (CSSR) | Conformal PICA is an alternate to PICA forebody TPS with improved mass efficiency |