Nanopore Sequencing-Based Microbial Air Profiling Method for Crewed Spacecraft

Brandon Dunbar1, Sarah Stahl-Rommel2, Hang N. Nguyen2, Christian L. Castro2, G. Marie Sharp3, and Sarah L. Castro-Wallace4

1GeoControl Systems, Inc., Houston, TX 77054, 2JES Tech, Houston, TX 77058, 3KBR, Houston, TX 77058, 4NASA Johnson Space Center, Houston, TX 77058

Microbial monitoring of the International Space Station (ISS) atmosphere is vital to maintaining the health of the spacecraft and crew. Key to NASA’s microbial risk assessment is the identity of contaminating microorganisms in the environment. Historically, this has been achieved through impaction-based air sampling followed by culture. Identification of the microorganisms present requires sample return to Earth and lab-based analyses. While this culture-based approach has served to provide alerts to anomalies and overall confidence in the controls in place, it is not suitable for exploration missions with no sample return. Recently, significant advancements in molecular-based microbial monitoring via nanopore sequencing have been implemented onboard the ISS. Building on this work, multiple commercially available air samplers, compatible with downstream molecular analysis, were evaluated for use in the spaceflight environment. Through this assessment, the Coriolis Compact (Bertin Technologies), which uses cyclonic technology to collect bioaerosols onto the surface of a sterile cone, was selected for a larger-scale comparison to the current culture-based monitoring method. Using the Coriolis Compact, 1000 L of air was collected from the breakroom of an office building and a fitness center. The buffer used to dissociate the microbial cells from the surface of the cone was split between the NASA Microbiology Laboratory’s standard culture and Sanger sequencing-based method and the culture-independent nanopore sequencing method. The bacteria identified through culture were present in the nanopore data, with Micrococcus, Staphylococcus, and Moraxella being the most common cultured isolates, which is expected based on the media and growth conditions. Not surprisingly, the nanopore data yielded much higher diversity and paralleled that of previous atmospheric microbiome studies of human-occupied built environments. As compared to the culture-based data where the breakroom and fitness center data sets cluster in proximity, the nanopore data depicts the contrast of these atmospheric microbiomes. Moreover, the nanopore data were sufficient to meet NASA’s risk assessment needs and noted the culturable isolates routinely observed. This nanopore-based atmospheric microbial profiling method will enable near real-time environmental monitoring of crewed spacecraft as future missions extend beyond low-Earth orbit.