Recent Advancements in Electrical Capacitance Mass Gauging for Cryogenic Propellant Tanks

Chris Biagi1, Mark Nurge1, Robert Youngquist1, Jedediah Storey1, Luke Bird1, Austin Atkins1, Adam Swanger1, Nickolas Spangler1, Mariano Mercado2, Jesse Defiebre2, Jason Hartwig2

1NASA Kennedy Space Center, USA

2NASA Glenn Research Center, USA

ABSTRACT:

The current lack of unsettled mass gauging is a key roadblock for many space activities, such as orbital refueling, missions to the Moon and Mars, and nuclear thermal and nuclear electric propulsion technologies.  Liquid can form any one of an infinite number of configurations in microgravity, such as floating in globs or accumulating on tank surfaces in discontiguous volumes, or both.  Capacitive sensing requires no moving parts and dissipates close to zero heat, making it an ideal candidate for cryogenic fluid mass gauging in settled and unsettled configurations. Capacitive sensing has a history of proven use in space propellant tanks, including tanks on the space shuttle, Saturn V, and the Apollo Lunar Excursion Module. Modern capacitive sensor technology allows using the entire tank as the capacitor volume by placing electrodes on the tank walls and propellant management surfaces. Capacitance is directly related to density, and therefore to mass for fixed volumes.  In this presentation, we discuss the concepts behind whole-tank capacitance mass gauging and associated engineering challenges.  We describe recent efforts to develop a micro-g unsettled cryogenic mass gauge using whole-tank capacitance sensing, including the development of test beds, electronics, and algorithms. We describe several mathematical processing techniques, including empirical-based averaging, electrical capacitance volume tomography, and spatial regularization.  A modeling study, performed using settled configurations in gravity and no gravity, and with a set of 100 random fluid configurations, has indicated that spatial regularization, in which capacitance measurements are weighted to account for non-uniform electric fields, yields a mass fraction accuracy of 8% for any fluid configuration. Such a sensor is expected to operate in real time with a sampling frequency of at least 1 kHz.

Dr. Chris Biagi

NASA Kennedy Space Center

Christopher.j.Biagi@nasa.gov

PH: