Mitigating Microbial Contamination Risks in CO2 Removal Systems for Long-Duration Space Missions

A. Nicolas Whitlock, Pranav Jagtap, Tiago F. Costa

KBR, NASA Ames Research Center, Moffett Field, CA, 94035-1000

Jessica A. Lee, Grace A. Belancik

KBR, NASA Ames Research Center, Moffett Field, CA, 94035-1000

Crewed missions to Mars and beyond pose new challenges for the crew and the technological solutions they depend upon. Crucial considerations in designing the next generation of Environmental Control and Life Support Systems (ECLSS) include enhanced reliability and autonomy, given the challenges of resupply and resource recovery in remote missions. One significant challenge faced in various sectors of the International Space Station and ECLSS is the phenomena of microbial growth, particularly biofilm formation. As such, microbial growth is a critical consideration in the design phase of new ECLSS systems. Two novel CO2 removal systems—Liquid-Amines CO2 Removal (LACR) and Cold-Surface CO2 Deposition (CDep)—are currently under development at the NASA Ames Research Center to meet the needs of extended space travel. In this presentation, the authors aim to make a risk assessment of microbial growth in LACR and CDep through: 1) a comprehensive literature review to identify LACR and CDep components most vulnerable to microbial contamination and 2) wet lab testing commencing with a subscale test of the most susceptible component of CDep. The literature review revealed that the hollow-fiber Liqui-Cel membrane employed in CDep exhibits the highest susceptibility to microbial growth. Our wet lab tests corroborated these findings, as airborne microbes introduced into the membrane led to considerable biomass accumulation. These results indicate a significant risk of microbial invasion into the membranes. To prevent microbial growth in CDep or other CO2 removal systems, proper ECLSS arrangement is essential—a CO2 removal system should remain behind the charcoal-HEPA filters, condensing heat exchanger, and Trace Contaminant Control System. In summary, anticipating potential microbial interactions within ECLSS for remote crewed missions is vital to mission success. It should, therefore, be included as one of the many factors when designing future ECLSS systems to perform reliably and consistently to support astronaut life.