Decoding the energy inputs that drive the upper atmosphere with NASA’s Geospace Dynamics Constellation (GDC)

Eftyhia Zesta1 , Denny Oliveira6,1, Tom Sotirelis2 , Aaron Ridley4, Phil Anderson5, Rod Heelis5, Daniel Gershman1, Marilia Samara1, Mark Moldwin4, Meghan Burleigh7, Mike Hartinger8, Jorg-Micha Jahn3, Robert Michell1, George Khazanov1, Alex Chrartier2

(1) NASA Goddard Space Flight Center

(2) Johns Hopkins University Applied Physics Laboratory,

(3) Southwest Research Institute,

(4) University of Michigan

(5) University of Texas, Dallas

(6) University of Maryland Baltimore County

(7) Naval Research Laboratory

(8) Space Science Institute

NASA’s Geospace Dynamics Constellation (GDC) mission will study the effect of space weather phenomena on Earth’s upper atmosphere ion and neutral gases and, in turn, their impact on human assets. GDC will provide multi-point observations to determine how the Earth’s upper atmosphere is driven to dynamic changes and extremes by the solar wind.  Magnetospheric energy input, both kinetic and electromagnetic, will be captured by the Comprehensive Auroral Precipitation Experiment (CAPE), the Thermal Plasma Sensor (TPS), and the Near Earth Magnetometer Instrument in a Small Integrated System (NEMISIS). CAPE captures the complete kinetic energy inputs with observations of the precipitating and upgoing electrons and precipitating ions. TPS captures part of the electromagnetic energy inputs with observations of the thermal plasma velocity that map E-field inputs, and NEMISIS captures the other part of the electromagnetic energy input with measurements of magnetic field variations due to field aligned currents. We use similar observations from the Defense Meteorological Space Platform (DMSP) satellites to demonstrate the balance between the different energy inputs (electron and ion Joule heating and Poynting flux) during the different phases of storms. We explore the local time and spatial structure and the impact they have on atmospheric density and winds with simulations of the Global Ionosphere and Thermosphere Model (GITM) and demonstrate how GDC will enable us to reconstruct those energy inputs locally and regionally.