Integrated In-Situ Resource Utilization Modeling of a Lunar Water Processing System

Avery Carlson, Noah Andersen, Ashton Archer   -- Jacobs Technology, Inc.

Jacob Collins, Thomas Chen, Jeff Michel, Desmond O’Connor  -- NASA Johnson Space Center

Abstract:

A key element of achieving a sustained surface presence, such as defined in NASA’s Artemis plan, is In-Situ Resource Utilization (ISRU). ISRU is the practice of using local resources to provide mission consumables that reduce system launch mass requirements, and regenerate resources (chiefly, water and oxygen) for propulsion and life support supporting both Lunar and Martian missions. ISRU systems require multiple complex processes, such as excavation, chemical reactors, and electrolysis subsystems that must operate in harmony to optimize the overall system process from beginning to end. The Mission Analysis and Integration Tool (MAIT) connects individual subsystem models into a customized, flexible framework for the purpose of technology downselect, optimization, and end-to-end process planning. MATLAB was favorable to use as the main software integration tool due to its ability to communicate with a vast number of other programming languages and makes up the backbone of data flow between inputs and outputs to the subsystem models. MAIT initially evaluated a suite of technologies, including but not limited to, the water processing Lunar Auger Dryer for ISRU (LADI) system and an oxygen extraction carbothermal reduction process. With individual models consolidated, the MAIT tool generated over 60,000 cases during its parametric sweeps; these system iterations produced valuable insight into the optimal LADI geometry for minimizing heater energy demands, estimating carbothermal reactor and radiator mass relationships, and calculated the power dynamics of the electrolysis unit. Efforts to update the MAIT tool are ongoing to accommodate and scale ISRU technologies supporting the Space Technology Mission Directorate’s (STMD) commercialization strategy, and increase the MAIT software capability to handle a wide array of ISRU system models beyond the Lunar environment, e.g. production of propellant for a Martian lander.