Comparison of Anthropomorphic Test Device and Human Volunteer Responses in Landing Impact Tests of U.S. Space Vehicles

Teresa Reiber1, Preston Greenhalgh1, Keegan Yates1, Rachel Thompson1, Aaron Drake1, Nathaniel Newby1, Dustin Gohmert2, Jeffrey Suhey2, Chris Perry4, John Buhrman4, Jeffrey Somers2, Cynthia Null3

1 KBR, Houston, TX

2 NASA JSC, Houston, TX

3 NASA Ames, Mountain View, CA

4 AFRL, Wright Patterson AFB, Dayton, OH

United States (U.S.) crewed vehicles are being designed to support the National Aeronautics and Space Administration’s (NASA’s) human spaceflight programs. Vehicles must be designed to meet NASA’s occupant protection requirements including landing injury assessment with anthropomorphic test devices (ATDs) and analytical models. However, these tools are limited in capturing all injuries that might occur during spacecraft landings. A NASA study of injuries during Soyuz vehicle landings has shown that analytical models are underpredicting occupant injury. Because of the inherent limitations with our analytical tools, human volunteer impact testing was employed to validate the safety of U.S. crewed vehicles.

A total of 84 human volunteer tests in 11 different test orientations and g-levels were completed as part of this effort in collaboration with the Air Force Research Laboratory (AFRL) at Wright-Patterson Air Force Base and the vehicle development companies. Human subjects were tested at various realistic landing loads and in the highest fidelity seat and suit components that were available at the time of testing for two U.S. vehicles. Matched-pair ATD tests in the same test conditions were also conducted with small female and midsized male Hybrid III ATDs. ATDs were fully instrumented. Head accelerations and subjective responses were recorded in human subjects. In some cases, chest accelerations were captured.

Responses of the ATDs and humans in matched-pair tests were compared. No ATD tests showed evidence for risk of injury based on NASA occupant protection requirements. Human subjects reported 17 cases of discomfort or pain, 1 human subject was diagnosed with a minor injury that was not evident in the ATD tests. These results provide evidence that ATDs do not capture all potential injury risks, namely lower severity injuries, discomfort, pain, and fit issues. Overall, human testing is beneficial to understanding the true risk of injury to crewmembers during Earth landings.