Proposed Benchmark Problems for Validation of Low-G SPH Slosh Models

(Preprint) AAS XX-XXX

Eric Stoneking

GNC design needs to account for slosh dynamics that may interact with the GNC control system.  High-fidelity CFD simulations are too slow to incorporate into GNC design simulations for all but a few limited high-value cases.  There is a need for low-order slosh models, trading fidelity for speed, suitable for design work.  To ground these coarser models in the high-fidelity CFD results, a set of benchmark problems is needed.

This paper presents three artificial situations designed to showcase features of particular interest to low-gravity slosh modeling: a "dam break" in a tank in a one-g field, formation of an ullage bubble in zero gravity, and fuel interaction with a propellant management device (PMD) in zero gravity.  These problems lend insight into slosh dynamics excited by gravity, surface tension, and external acceleration transients.  If the same problems are simulated by a high-fidelity CFD facility, the results can be used to evaluate and validate candidate low-order slosh models, and facilitate tuning of model parameters.

As a first step, the dam break problem is presented using smoothed particle hydrodynamics (SPH) as a candidate low-order model.  Behavior may be qualitatively evaluated by animating the liquid-gas motion.  Quantitative metrics are recorded in the form of forces on the tank, location of the fluid mass center, and kinetic, gravitational potential, and internal energy terms.

Senior Engineer, Attitude Control Systems Branch, NASA Goddard Space Flight Center..